| Recapitulation Nico Boon, august 1993, update October 2003 Last revision: 1-3-2010 Handicap systems, a recapitulation 1 Empirical ? Theoretical ? Both systems combined ? Empirical : Measured sailed times......result in yardsticknumbers. Theoretical : Measured dimensions.....result in ratingnumbers. 2. Yardsticknumbers used since 1960....... for monohulls and multihulls. Since 1982 in Holland (and elsewhere) chosen for ratingnumbers. Why ? There are many new designs and not enough data for accurate statistics. 3. To test ratingnumbers it is recommended to draw so-called "Box and Whisker" diagrams. Only 6 numbers used : total number of boats, the time of the first, the last, the middle one and boats at the ends of the middle 50 %. 4. Fundamental difference between a yardstick and a rating system. • Yardsticknumbers determined by using.....ALL.....elements of speed. • Ratingnumbers determined by using.....SOME....elements of speed.. 5. There are 20 elements of speed, according to Olin Stephens (1972). 1. length 2. sailarea 3. displacement 4. prism. coeff. 5. asp. ratio rig 6. ratio main/jib . . 7. beam 8. wetted surface 9. centre of effort 10. draught 11. ballast 12. combined 1 - 12 13. fullness ends 14. weight distribution 15. freeboard 16. windage rig 17. motor 18. material 19. course stability 20. steering stability 6. There are 7 elements of speed which have nothing to do with the design. 1. helmsman 2. crew 3. organisation 4. condition of sails 5. condition of bottom 6. tactics / strategy 7. luck 7. Follow the "KISS" principle, "keep it simple stupid", for formulas. There are 4 formulas available to approximate the performance. Bruce nrs (1967), IOMR (1968), KSP since 1976, Texel Rating since 1984 Edmund Bruce, pioneer of Amateur Yacht Research Society, died 1973. • IOMR = International Offshore Multihull Rule (California, 1968). • KSP = Kelsall Shuttleworth Performance. (MOCRA formula, around 1978). • TR = Texel Rating by Nico Boon, 1984, slightly modified in 1993. 8. IOMR, KSP and TR use length, sailarea and weight (displ.) only.. RL = rated length, RSA = rated sailarea, RW = rated weight. (kg.). General form of these formulas : 100 / (c * RL ^ p * RSA ^ q / RW ^ r). • Bruce (metr.) =2.5209 * RL ^ 0 * RSA ^ 0.5 / RW ^ (1/3 ). • Bruce (imp. ) = 1 * RL ^ 0 * RSA ^ 0.5 / RW ^ (1/3 ). • KSP (metr.) = 100 / (2 * RL ^ 0.5 * RSA ^ 0.5 / RW ^ 0.5 ) . • KSP (imp. ) = 100 / (0.5 * RL ^ 0.5 * RSA ^ 0.5 / RW ^ 0.5 ). • MOCRA (metr.) = 100 / (1.74 * RL ^ 0.2814 * RSA ^ 0.3546 / RW ^ 0.3124) • MOCRA (imp. ) = 100 / (0.6865 * RL ^ 0.2814 * RSA ^ 0.3546 / RW ^ 0.3124) • TR (metr.) = 100 / (1.15 * RL ^ 0.3 * RSA ^ 0.4/ RW ^ 0.325) • TR (imp. ) = 100 / (0.4024* RL ^ 0.3 * RSA ^ 0.4 / RW ^ 0.325). * TR (metr.) = 100 / (1 * RL ^ 0.3 * RSA ^ 0.4/ RW ^ 0.325) * TR (imp.) = 100 / (0.3499 * RL ^ 0.3 * RSA ^ 0.4/ RW ^ 0.325) • IOMR (metr.) = 100 / (1 *RL ^ 0.311 * RSA ^ 0.42 / RW ^ 0.318). • IOMR (imp. ) = 100 / (0.3275 * RL ^ 0.311 * RSA ^ 0.42 / RW ^ 0.318). • IOMR here, is a "translation" into a power formula of the IOMR formulas. 9. Above Beaufort 5 or 6, stability of a design is the limiting factor for speed. A rating has to be calculated then by using the smaller sailarea making heeling moment = righting moment. No other formula required... |  |
| --- | --- |